# DARTalyze

# Overview

DARTalyze are 2 python based scripts that produce either:

- Extracted ion chromatograms of targeted ions analyzed by DART in excel format.

- The largest ‘base peaks’ which occur in the raw file

This script relies on freely available ‘Anaconda modules’ to monitor the intensity of multiple ions over time

# Requirements

- Install anaconda (https://docs.continuum.io/anaconda/install) onto PC that will be used to analyze .raw data

- Install Proteowizard (http://proteowizard.sourceforge.net/downloads.shtml) in order to convert the Thermo® .Raw files into text based files

- Required files

  - DARTalyze.py

  - dtyc..(key file)

# Building working directory

- Make a folder on Desktop which will be the working folder

- In this new folder,

  - make a folder called ‘raw files’

  - Make 2 folders: Nontargeted and Targeted

  - Paste/move/download “Target_DARTalyze.py” and Target_input into the Targeted

  - Paste/move/download “Non-Target_DARTalyze.py” and Non-Target_input into theNontargeted folder

# Converting the files

Convert the Thermo® .raw file to a ‘text’ file using Proteowizard (MSconvert). Use the ‘peak picking’ filter

- Browse in the raw file(s)

- Click ‘Add’

- Select the output folder. Preferably, into the ‘raw files’ folder of the DARTalyzer script

- Select ‘text’ under Output forma

- Choose ‘Peak Picking’ under the Filters option

- Click the ‘Add’ button below

- Click Start

Move the converted file into the ‘raw files’ folder of the DARTalyzer

# Running DARTalyze

### Input file

Open the ‘input.txt’ file in a text editor (i.e notepad)

Below the prompts, input the: raw file name, mass accuracy, output file,

Input the top X ion m/z that will be monitored.

Below the ‘List all masses to be excluded’ line, record the m/z of all ions to be excluded.

Save the file

### Run the file

Double click on the “DARTalyze.py” file

- A new “output.xlsx” file should appear containing the data

- Open the output file (.xlsx) to view the data.

- The data can now be visualized using standard excel practice